TAP-std working group                                       D. Bernstein

RFC NNNN                                                              IR

June 1991, revised 17 August 1992

TAP

Status of this Memo

This memo provides information for the Internet community. It does

not specify an Internet standard. Distribution of this memo is

unlimited.

1. Introduction

It is common for Internet hosts to associate relatively long-lived

information to each TCP connection, often (but not always) including

a ``username'' or ``owner name'' or some other information about the

entity using the connection. TAP announces the information associated

with a particular TCP connection to the host on the other end of the

connection. TAP may be used on any host which associates relatively

long-lived information to each connection.

2. Overview

This is a connection-based application which runs over TCP. The TAP

server listens for TCP connections on port 113 (decimal). After a

connection is established, the server reads one line of data which

specifies the connection of interest. If that connection exists and

is associated with system-dependent information, the server sends

the information. Otherwise it sends an error line. After sending the

information or error line, the server closes its connection. After

reading the information or error line, the client closes the

connection.

The server will give information about TCP connections between the

server's host and host H only to host H itself. The two hosts (i.e.,

IP addresses) involved are not transmitted explicitly by the

protocol; they are implicit in the connection made to the server.

3. Request format

The server accepts simple text query requests of the form

<localport> , <foreignport>

TAP-std working group                                           [Page 1]

RFC NNNN                         TAP                         August 1992

where <localport> is the TCP port on the server's host and

<foreignport> is the TCP port on the client's host. All numbers are

expressed in decimal without a sign, and all text is ASCII. If the

request is not in this format, the server may immediately drop the

connection.

For example, say rose is connected to the standard TELNET port on

host tulip, through TCP ports 6191 on rose and 23 on tulip. (Note

that rose and tulip are simply names used in this document to

identify two IP-connected machines. They are not fully qualified

domain names.) tulip connects to the TAP server at port 113 on rose.

It sends this line:

6191 , 23

Here 6191 is the TCP port on the TAP server's host (rose) and 23 is

the TCP port on the TAP client's host (tulip). This uniquely

specifies the given TELNET connection.

The precise format of the request line is as follows: <localport>,

followed by any amount of whitespace, followed by a comma and any

amount of whitespace, followed by <foreignport>, followed by carriage

return and line feed. Whitespace means space or tab; "any amount"

means zero or more, though a client should not print excessively many

spaces. The server should read until the line feed and respond

immediately. The client should not send anything after the line feed,

though future revisions of this specification may permit additional

data. The client should not add initial zeros to its decimal numbers,

but the server must accept such numbers. Future revisions of this

standard may assign additional meaning to decimals with a leading 0.

4. Response format

The server sends a response line in one of these two formats:

<localport> , <foreignport> : USERID : <systemtype> : <conn-info>

or

<localport> , <foreignport> : ERROR : <errortype>

Here <localport> and <foreignport> are the same numbers as in the

query. (If the client uses initial zeros, the server may do so as

well, but otherwise it should not use initial zeros.) <systemtype> is

an operating system name for the server's host as described in

Assigned Numbers, RFC 1060 or its successors. <conn-info> is

system-dependent information associated to the connection.

<errortype> is text describing an error as outlined below.

TAP-std working group                                           [Page 2]

RFC NNNN                         TAP                         August 1992

<systemtype> may also be OTHER to specify any other operating system

not yet listed in Assigned Numbers. Even if the server's system is

listed in Assigned Numbers, the server may use OTHER for any reason,

including operating system type privacy. Future revisions of this

document may permit further values of <systemtype>.

<conn-info> is in some format defined by the system. This document

does not define the format or meaning of <conn-info>. Often

<conn-info> is in the same format as a system-dependent mailbox name,

which is typically in the same format as a system-dependent username,

but these equivalences are not required. <conn-info> could be

encrypted with a secret key; it could carry something other than

information about the entity using the connection. For the purposes

of this protocol, <conn-info> is an uninterpreted octet string. See

section 5 for further details.

For example, some possible responses to the 6191 , 23 query might be

the following:

6191 , 23 : USERID : UNIX : joe

6191 , 23 : USERID : MULTICS : StJohns.DODCSC.a

6191 , 23 : USERID : OTHER : StJohns.DODCSC.a

6191 , 23 : USERID : TAC : MCSJ-MITMUL

6191 , 23 : USERID : UNIX : a6X#-Yp,3147,2910

6191,23     :USERID:OTHER:wewishyouamerrychristmasandahappynewyear

6191 , 23 : ERROR : NO-USER

An ERROR line means that the server could not determine the

information associated to the TCP connection. <errortype> tells why.

<errortype> may be any of the following:

INVALID-PORT

<localport> or <foreignport> was improperly specified---out of

the range 0 to 65535, for example---or the request was

otherwise non-standard. In this case the server may drop the

connection without replying.

NO-USER

The connection specified by the port pair is not currently in

use.

HIDDEN-USER

The connection is in use, but the information associated to it

is explicitly hidden.

TAP-std working group                                           [Page 3]

RFC NNNN                         TAP                         August 1992

UNKNOWN-ERROR

Cannot determine the information associated to the connection,

for an unknown reason. The server may give this <errortype> in

any case and for any reason, including privacy, whether or not

another <errortype> applies.

Future revisions of this document may allow other <errortype> values.

The server may also report an <errortype> beginning with the letter

X; all such <errortype>s are reserved for experimental or

non-standard use.

The precise format of the response line is as follows: <localport>,

followed by any amount of whitespace, followed by a comma and any

amount of whitespace, followed by <foreignport>, followed by any

amount of whitespace, followed by a colon and any amount of

whitespace. In the USERID case, it is then followed by USERID and any

amount of whitespace, a colon and any amount of whitespace,

<systemtype> and any amount of whitespace, a colon and any amount of

whitespace, one or more characters giving <conn-info>, and finally

carriage return and line feed. In the ERROR case, it is followed by

ERROR and any amount of whitespace, a colon and any amount of

whitespace, one or more characters giving <errortype>, and finally

carriage return and line feed.

Note that this format is ambiguous if <systemtype> contains colons or

whitespace. Assigned Numbers does not currently list any <systemtype>

with colons or whitespace, but if it ever does, the TAP server must

use OTHER for the <systemtype> on such a machine. The server should

also not use a <systemtype> containing carriage return or line feed.

Similarly, if <conn-info> or <errortype> begins with whitespace or

contains carriage return-line feed, the response line format is

ambiguous. The server must never use <errortype> containing

whitespace, carriage return, or line feed, and future revisions of

this RFC will never provide for such an <errortype>. The server

cannot send <conn-info> beginning with whitespace or containing

carriage return-line feed; it should not send <conn-info> containing

whitespace, carriage return, or line feed. ERROR : UNKNOWN-ERROR is

preferable.

Finally, <systemtype>, <conn-info>, and <errortype> cannot be empty

strings, and cannot contain ASCII NUL (character 0).

TAP-std working group                                           [Page 4]

RFC NNNN                         TAP                         August 1992

Later revisions of this protocol specification may further restrict

the octets which may be transmitted. In light of this, servers

should, if possible, limit <conn-info> to at most ASCII codes 33

through 126. Clients should, however, be prepared to handle all

octets.

Note that there is no limit on line lengths: in particular, on the

length of <conn-info>. The client may drop the connection at any

time to avoid overflow. The server should, if possible, place the

most useful information within the first 512 characters of

<conn-info>.

5. Applications and security

A TAP server may place any information it wants into its responses to

a TAP query. This protocol does not assign any meaning to <conn-info>

beyond its intrinsic existence as an octet string. So, in most cases,

a TAP client will simply record the bytes of <conn-info> in some

manner for possible interpretation later by the server host. This is

primarily useful as a form of remote auditing: if the client host

judges that the TCP connection represents an accidental or malicious

malfunction on the part of the server host, then <conn-info> may

permit the server host's owner to track down the exact source of the

malfunction. So the information returned by TAP in this case is of

primary benefit to the host generating that information.

The TAP client can do nothing more than this unless it has an

external reason to assign meaning to <conn-info> received from that

particular host. Beware that assigning <conn-info> an unjustified

meaning will in general lead to security holes. Do not use TAP for

access control without documenting the external knowledge you have

which ensures that your use of <conn-info> for access control is

justified. An attacker could subvert the security of a server host or

of TCP/IP; and any host could send any information it wants along a

TAP connection. Two dangers of weak methods of access control are

that they may permit access which should be denied, and that they may

deny access which should be permitted.

6. Notes

This section is not part of the TAP description proper. It provides

historical information and pointers to further information.

TAP-std working group                                           [Page 5]

RFC NNNN                         TAP                         August 1992

TAP is derived from the protocol defined in RFC 931 by Mike StJohns.

It was first implemented by this author in early 1990, then again in

February 1991, and distributed via the USENET network under the name

authd. Later in 1991 two more independent interoperable

implementations were distributed through the Internet. In 1992

another independent interoperable implementation was distributed.

TAP, as defined in this document, is the same as the authd protocol,

which has not changed since its first implementation in early 1990.

This document is a cooperative effort of the TAP-std working group.

TAP-std is an ad-hoc group with the following charter: ``This group

is chartered to document the TAP protocol as used on TCP port 113

around the Internet. Its first goal is to publish as quickly as

possible an accurate, well-understood, complete TAP specification

which reflects the consensus of the community. Afterwards it will

keep track of TAP usage and produce further documents as necessary.

This group will not publish as a standard any TAP variation which has

not been tested on the Internet, though it will make all reasonable

allowances for future extensions.''

At the time of publication of this document, you can join the

tap-std mailing list by sending a subscription request to

tap-std-request@kramden.acf.nyu.edu, or by connecting to TCP port

39311 on 128.122.142.2 and typing your username. There is also a

mailing list for people who want to use TAP to solve problems. To

join, send mail to rfc931-users-request@kramden.acf.nyu.edu.

The author would like to thank Chris Davis, Peter Eriksson, and Dave

Borman for their helpful suggestions used in creating the TAP-std

base document. Thanks go to all the participants in the TAP-std

working group for their suggestions, comments, and criticism.

Security Considerations

Security issues are discussed in section 5.

Author's Address

Daniel J. Bernstein

5 Brewster Lane

Bellport, NY 11713

Email:  brnstnd@nyu.edu

TAP-std working group                                           [Page 6]