Object Type:	tabgate

Description:	A gate with tabulated dependencies on any field for

opening and closing.

Author:		U. S. Bhalla, Caltech (2/90)

------------------------------------------------------------------------------

ELEMENT PARAMETERS

DataStructure:	tab_gate_type  [in src/olf/olf_struct.h]

Size:		96 bytes

Fields:		activation	input field, usually accessed by messages

m		state of gate

alpha		rate variable table filled by TABCREATE

alpha_alloced	internal flag for table allocation

beta		rate variable table filled by TABCREATE

beta_alloced	internal flag for table allocation

instantaneous	flag for looking at instantaneous response

------------------------------------------------------------------------------

SIMULATION PARAMETERS

Function:	TabGate  [in src/olf/tabgate.c]

Classes:	segment

gate

Actions:	PROCESS

CHECK

RESET

SET

SAVE2

RESTORE2

TABCREATE

TABFILL

CALC_MINF

CALC_ALPHA

CALC_BETA

Messages:	VOLTAGE compartment-voltage

SUM_ALPHA value-to-sum-onto-alpha-func

PRD_ALPHA value-to-multiply-alpha-func-by

SUM_BETA value-to-sum-onto-beta-func

PRD_BETA value-to-multiply-beta-func-by

EREST resting-potential-for-resets

Notes:		A gate with tabulated dependencies on any variable for opening

and closing. Voltage and ion concentration are the most common

variables for dependencies.  The tabgate object is very

similar to the vdep_gate object, except that it uses tables

for the voltage and other dependencies. It also has message

slots for additional dependencies.  It is normally used to

send MULTGATE messages to a vdep_channel.  However, this

combination has been largely supplanted by the tabchannel

object.

A gate state is calculated from the following differential

equation:

dm/dt = alpha(1 - m) - beta * m

The alpha and beta functions are implemented as tables and

therefore have to be explicitly allocated using the TABCREATE

function. Copies of tabgate use the original arrays unless new

arrays are explicitly allocated.

The TABCREATE function

TABCREATE here is different from the table element since we

need to specify which table to create:

call <element> TABCREATE <alpha/beta> <xdivs> <xmin> <xmax>

This call creates an array on alpha or on beta with xdivs

entries, which spans the range xmin to xmax. Values outside

this range are linearly extrapolated using the nearest 2 table

entries.

The TABFILL function

TABFILL expands the table by using interpolation on the

existing table entries.  The syntax is different from the

table element, as one has to specify the array (alpha or beta)

being filled:

call <element> TABFILL <alpha/beta> <xdivs> <fill_mode>

xdivs + 1 specifies the number of entries that the expanded

tables should have.

fill_mode is one of

0 = B-spline fill (default)

1 = cubic spline fill (not yet implemented)

2 = Linear fill

Scaling tables

When the fields alpha->sx, alpha->sy, alpha->ox, alpha->oy,

and the corresponding fields on beta are set, the table

element automatically scales on the entries in the

interpol_struct array so as to avoid scaling computations

during simulations.

The CALC_ALPHA, CALC_BETA, and CALC_MINF functions

The CALC_ALPHA, CALC_BETA, and CALC_MINF actions are used with

calls of the form

y = {call <element> CALC_ALPHA  <x-value>}

In order to access values of alpha, beta or m_inf (the steady

state activation, alpha/(alpha + beta)) for a particular value

of the dependent variable.  This feature is most often used

for making plots of these quantities.  Note that the

activation time constant can be calculated from tau = 1/(alpha

+ beta).

Example:	see Scripts/neurokit/prototypes/traub91chan.g

See also:	Interpol documentation (Tables), tabchannel, vdep_channel